HIGH-SPEED ROLL-UP DOOR, type “EFA-SRT®-MTL”

Manufacture, delivery and installation of

High-speed roll-up door, type „EFA-SRT®-MTL“, with electro-mechanical high-performance drive unit for inside use without wind load as “access prevention” in materials logistics without personal traffic.

The main advantage of this low-maintenance door construction consists of the very slim aluminium frames (anodized (E6-EV1) with clinging door blade guides. Standard door curtain made of 2mm thick PVC, fully transparent, laterally guided and rolled onto horizontally placed shaft; PVC curtains generally come with vertical warning stripes.

The DOOR is driven by a high-frequency motor as direct drive. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches, with the limits being determined electronically. Electro-mechanical limit switches are not permissible here.

OPENING SPEED:	up to approx. 1.5 m/s

CLOSING SPEED:	up to approx. 0.6 m/s

(depending on door size)

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate plastic switch cabinet, protection type IP 65 with CEE plug. Connection to 230V / 50 Hz power supply on-site.

The scope of delivery includes a bottom section for curtain tensioning made of galvanized steel with sealing lip.

Regulations pursuant to DIN EN 13241-1 are complied with;

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com