OPTIONS for high-speed roll-up door, type “EFA-SRT®-L EX”

Lining of the winding shaft/motor:

Complete covering over motor and shaft

Surface:

Powder coating of all galvanised steel parts in a colour according to RAL __________

or

Stainless steel version (V2A) of all visible steel parts, visible surface polished, coarseness 220,

incl. switch cabinet made of V2A

Activator:

- Push button in ATEX version, on-wall industrial version

- Push button combination OPEN/STOP/CLOSE ATEX version, on-wall industrial version

- Pull switch in ATEX version, optionally with wall-mount console

- Induction evaluation-unit in ATEX version (1 or 2 channel version)

(incl. cutting and loop laying)

Special EX-proof versions:

Door system according to ATEX-guidelines RL 2014/34EU for dust-ATEX-protection:

- EX-area-Zone 21 (II 2D IIIB 135°C X)

- EX-area-Zone 22 (II 3D IIIB 135°C X)

(X: Temperature range-15°C to+50°C)