HIGH-SPEED ROLL-UP DOOR, type “EFA-SRT®-L EX”

Manufacture, delivery and installation of

High-speed roll-up door, type „EFA-SRT®-L EX“, with electro-mechanical high-performance drive unit for permanent industrial application.

For the weight balancing of the door leaf and for manually opening the door in case of power failure (according to DIN EN 12604), a special tension spring mechanism is to be integrated into the lateral frame. This durable and extremely maintenance-friendly counter acting system is obligatory. Constructions using torsion springs are not admissible; standard door leaf made of black, not transparent, 1.4 mm thick ATEX-curtain, laterally led and winded onto horizontally supported shaft;  A maintenance-free door leaf tension must be provided in order to permanently keep the door leaf tensioned with at least 800 N. In addition, the lateral CURTAIN GUIDES with special guiding devices are to be developed so that a flawless winding up and unwinding is also ensured during wind load. Steel frame construction, completely sealed, sendizimir-galvanised as standard.

The DOOR is driven by a geared motor which has to be developed as high-frequency motor. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches (zone 1), or absolute encoder	with the limits being determined electronically. 
Electro-mechanical limit switches are not permissible for this.

OPENING SPEED:	up to approx. 1.0 m/s

Max. DOOR LEAF SPEED:	up to approx. 0.5 m/s

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate plastic switch cabinet, protection type IP 65. Connection to 230V / 50 Hz power supply on-site. It’s MANDATORY that the control-unit is installed outside of the EX area !

Door-safety:

- Electric Safety contact edge, self-monitoring acc. to DIN EN 12453 in EX proof version

Its connection cable must be guided in a protective energy chain within the door frame.

-	Safety photocell in EX proof version, protectively integrated in the door frame

-	Emergency-STOP-button in ATEX version (On-wall industrial version)

EX-proof versions:

According to ATEX-guidelines RL 2014/34EU the door system is suitable to be used in:

- EX-area-Zone 1 (II 2G IIB T4 X)

- EX-area-Zone 2 (II 3G IIB T4 X)

(X: Temperature range-15°Cto+50°C)

Any electrical component (excluding control-unit) will have a ATEX-certificate.
The mechanical EX-parts will be supplied according to ATEX-zone.

Regulations pursuant to DIN EN 13241-1 are complied with;

Air permeability to DIN EN 12426, class 0

(values depend on the door size and the equipment)

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com