OPTIONS for high-speed turbo door, type “EFA-STR®”

Surface

Powder coating of all galvanised steel parts in a colour according to RAL __________

(pearl, luminous and metallic colours not possible)

or

stainless steel version (V2A) of all visible steel parts, visible surface polished, coarseness 220,

incl. switch board made of V2A, incl. guide rollers with V2A bearings, e.g. for wet operations

Standard speed:

Reduced price for version with standard speed:

OPENING SPEED:	up to approx. 1.6 m/s

Max. DOOR LEAF SPEED:	up to approx. 2.0 m/s

(depending on the lifting height)

CLOSING SPEED:	up to approx. 1.0 m/s

Fixed side element:

Surcharge for fixed lateral element consisting of:

Integrated pedestrian door, DIN (to the left and to the right of the door) with lock and set of door handles (on-site closing cylinder)

clear passage max. B = 1,500 mm, H = 2,500 mm

Above-fixed cover element incl. required frame structure, similar to door leaf with a total size of ca. B = 	mm, H = 	mm

OPTIONS:	-	Panic lock for pedestrian door

-	Overhead door closer

-	Closing cylinder with 3 keys

Crash version:

Version of the door with "ACS-DS" (Active Crash System) by interlinked hinge changes which are detachably connected up to a height of 900 mm. Power transmission by direct synchronous drive and two linearly moved pistons. Crash recognition by inductive proximity switches integrated on both sides of the bottom section.

Restarting of the door optionally by:

Fully automatic retraction (standard)

Manual retraction by means of the membrane keypad or key switch (option)

IF REQUIRED:

Version with space-saving low lintel guidance

IF REQUIRED:

Automatic interim stop at a height of H = ___________ mm