OPTIONS for high-speed spiral door, type “EFA-SST®-L EX”:

COVER SHAFT / MOTOR

- Spiral cover bottom and front

- Spiral cover top (not for oval spiral-shape) as dust cover

- Motor cover

Surface

Powder coating of all galvanised steel parts in a colour according to RAL __________

or

Special coating of the laths in a colour according to RAL _______

If steel parts as well as laths parts be coated in the same RAL colour, minor deviations in colour may occur which can not be fully avoided due to the different surface structures. The supplier, however, will make the best possible efforts to keep deviations in colour as small as possible through influencing the degree of gloss.

Stainless steel version (V2A) of all visible steel parts, visible surface polished, coarseness 220,

incl. switch board made of V2A.

Door leaf

Option 1 lath separation 151 mm (standard double-walled, aluminium laths):

Surcharge for door leaf filling of single-walled, transparent acrylic glass.
Alternatively: Surcharge for door leaf filling of single-walled, transparent and impact resistant

polycarbonate.

Option 2 lath separation 225 mm (standard aluminium profiles with single-walled acrylic glass)

Surcharge for door leaf filling of single-walled, non-transparent acrylic glass (color:aluminium grey).

Alternatively: Surcharge for door leaf filling of single-walled, transparent and impact resistant

polycarbonate.

Activator:

- Push button in ATEX version, on-wall industrial version

- Push button combination OPEN/STOP/CLOSE ATEX version, on-wall industrial version

- Pull switch in ATEX version, optionally with wall-mount console

- Induction evaluation-unit in ATEX version (1 or 2 channel version)

(incl. cutting and loop laying)

Special EX-proof versions:

Door system according to ATEX-guidelines RL 2014/34EU suitable to be used in:

- EX-area-Zone 21 (II 2D IIIB 135°C X)

- EX-area-Zone 22 (II 3D IIIB 135°C X)

(X: Temperature range-15°Cto+50°C)