OPTIONS for high-speed door, type “EFA-SST®-PS”

Surface

Powder coating of all galvanised steel parts in a colour according to RAL __________

(pearl, luminous and metallic colours not possible)

Powder coating of the aluminium posts in a colour according to RAL _______

If steel parts as well as aluminium parts be coated in the same RAL colour, minor deviations in colour may occur which cannot be fully avoided due to the different surface structures. The supplier, however, will make the best possible efforts to keep deviations in colour as small as possible through influencing the degree of gloss.

Transparency:

Surcharge for sight lath with fully transparent, single-walled windows made of acrylic glass.

Optional:

Surcharge for sight lath made of impact resistant polycarbonate with scratch-resistant coating

Ventilation:

Surcharge for aluminium lath with single-walled perforated sheet metal.

Fixed side element:

Surcharge for fixed lateral element consisting of:

Integrated pedestrian door, DIN (to the left and to the right of the door) with lock and set of door handles (on-site closing cylinder)

clear passage max. B = 1,500 mm, H = 2,500 mm

Above-fixed cover element incl. required frame structure, similar to door leaf with a total size of ca. B = 	mm, H = 	mm

OPTIONS:	-	Panic lock for pedestrian door

-	Overhead door closer

-	Closing cylinder with 3 keys

Burglar protection:

Surcharge for a mechanical LOCKING MECHANISM. Operated by an inside hand lever.

Limit Switch:

Inductive limit switch, installed in a manipulation proof way in the door frame, for additional signal “DOOR CLOSED” to superior control system

Connection and evaluation on site

Alternatively:

Inductive safety limit switch (Cat IV), installed in a manipulation-proof way in the door frame, for additional signal “DOOR CLOSED” to superior control system

Connection and evaluation on site

IF REQUIRED:

Solution with space-saving low header version

IF REQUIRED:

Automatic interim stop at a height of H = ___________ mm