HIGH SPEED SPIRAL DOOR, type „EFA-SST®-LAS“

Manufacture, delivery and installation of

High-speed spiral door, type “EFA-SST®-LAS” in laser-tight version with electro-mechanical high-performance door drive as space-enclosing protective device against laser beams.

The door system mainly consists of:

Self-supporting lateral steel frames; steel parts generally galvanized, spiral-shaped door leaf guidance

The load is transmitted on both sides: For achieving this, a synchronised shaft will be installed. For the exact, smooth and low-noise guidance of the hinge straps, ball-bearing precision rolling units have to be used. A sufficiently dimensioned tension spring mechanism, ensuring the weight balancing of the door leaf according to the standard DIN EN 12604 and a manual opening of the door (e.g. in the case of a power failure) is installed in the door frames.

Door leaf made of double-walled aluminium laths which are fixed onto hinge straps and moved vertically (i.e. up or down). The laths are overlapping so that no direct penetration of the laser beams is possible. The surfaces of the aluminium laths are anodised E6/EV1.

This SPIRAL BODY is designed to guide the laths of the door leaf completely without contact and thus without wear and with best possible noise reduction.

The DOOR is driven by a geared motor which has to be developed as high-frequency motor. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches, with the limits being determined electronically.

OPENING SPEED:	up to approx. 1.0 m/s

Max. DOOR LEAF SPEED:	up to approx. 1.5 m/s

(depending on door size)

CLOSING SPEED:	up to approx. 0.6 m/s

The MICROPROCESSOR CONTROL is installed together with the frequency converter in a separate switch cabinet, protection type IP 65. Connection to 230V / 50 Hz power supply on-site.

The scope of delivery includes a DOOR LIGHT-LINE GRID (EFA-TLG®), TÜV certified and exactly effective in the door closing line: The safety system is integrated completely protected into the lateral frames and generates a very tight light grid of infrared beams up to a height of 2.5 metres. Obstructions are detected without contact, the automatic closing movement is stopped immediately.

The following directives and standards are complied with during the construction, the production and the assembly of the door, in particular the standards DIN EN 13241-1, DIN EN 60825-4, VDE 0837 part 4

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com