HIGH-SPEED SPIRAL DOOR, Type „EFA-SST® Therm Eco“

EFAFLEX offers the “EFA-SST® Therm Eco” high-speed spiral door for demanding industrial continuous operation. This door combines first-class performance with outstanding durability.

Technical features

- Self-supporting, galvanised steel frames with spiral door leaf support. Synchronised shaft for even force transmission. Ball-bearing precision roller assemblies for quiet running. A tension spring mechanism certified to DIN EN 12604 balances the weight of the door leaf and allows manual opening in the event of a power failure.

- Door leaf: Double-walled, anodised aluminium slats, which are fixed in hinge strips and move vertically. The surface has a 2-layer coating in white aluminium (similar to RAL 9006).

- Spiral body: disc guide completely contact-free - for low-wear and low-noise operation

- High-frequency gear motor brake with inductive proximity switches and electronic limit position control (without mechanical limit switches)

- Opening speed up to 1.3 m/s; closing speed up to 1.0 m/s

- EFA-TRONIC® with integrated frequency converter in plastic control cabinet (IP65), power connection 230V/400V at 50 Hz (on site)

- Integrated, TÜV-certified goal line light grid (EFA-TLG®) - contactless obstacle detection up to 2.5 metres high

performance values (depending on Equipment)

- Resistance to wind load: DIN EN 12424, up to Class 4

- Watertightness: DIN EN 12425, Class 3

- Air permeability: DIN EN 12426, Class 3

- Sound insulation: DIN EN ISO 717-1, up to 24 dB(A)

- Thermal insulation: DIN EN 12428, up to 1,4 W/m²K

dimensions of the clear opening

Width = ............... mm

Height = ............... mm