HIGH-SPEED SPIRAL DOOR LOW LINTEL,
Type „EFA-STR® -L N Essential“

EFAFLEX offers the „EFA-STR® -L N Essential high-speed spiral door, designed for demanding industrial continuous operation. The special door leaf guide system ensures minimal lintel clearance whilst providing optimal sealing in the lintel area.

Technical features

- Self-supporting, galvanised steel frames with a spiral-shaped door leaf mounting.

- Door leaf: 40 mm thick extruded aluminium profiles with a polyester fabric infill. The extruded profiles are spaced 225 mm apart.

- Spiral body: Fully contact-free vane guide – for low-wear and low-noise operation as a low-head variant requiring a particularly low head

- High-frequency gear motor with inductive proximity switches and electronic limit control (without mechanical limit switches)

- Opening speed up to 0.6 m/s; closing speed up to 0.5 m/s

- EFA-TRONIC® with integrated frequency converter in a plastic control cabinet (IP65), power supply 230 V/400 V at 50 Hz (to be provided by the customer)

- Integrated, TÜV-certified goal-line light curtain (EFA-TLG®) – contactless obstacle detection up to a height of 2.5 metres

performance values (depending on equipment)

- Resistance to wind load: DIN EN 12424, up to Class 4

- Water resistance: DIN EN 12425, npd

- Air permeability: DIN EN 12426, Class 0

- Sound insulation: DIN EN ISO 717-1, 12 dB(A)

- Thermal insulation: DIN EN 12428, up to 6,0 W/m²K

dimensions of the clear opening

Width = ............... mm

Height = ............... mm